Manual

for download at www.CamOnetec.com

Vers.: 1.02

CamOneTec is a Division of ACME the game company GmbH, Germany
"FlyCamOne", “CarCamOne” and "CamOne" are registered Trademarks of ACME the game company GmbH, Germany   
© copyright 2011 by CamOneTec

Safety advice

| WARNING: Follow these instruction to prevent damage of the unit: - DO NOT USE UNPROTECTED IF RAIN OR WATER. PREVENT CONTACT WITH ANY KIND OF LIQUIDS - USE RECOMMENDED ACCESSORIES ONLY - DO NOT DISASSEMBLE - THERE ARE NO PARTS AT THE INSIDE THAT NEED TO ME MAINTAINED - REPAIRS SHOULD BE MADE BY QUALIFIED PERSONNEL ONLY |  | SERVICE & SUPPORT CamOneTec by ACME Industriestr. 10a 33397 Rietberg Germany Tel: +49 5244 7000 70 service@camonetec.com or http:// www.camonetec.com |
| --- | --- | --- |

| WARNING Li-Ion BATTERY - Mishandling the battery may occur explosion, fire and smoke including damage of health or goods. It will also reduce battery performance. - Never charge a damaged Li-Ion battery pack. - Stop charging if the Li-Ion battery pack is getting deformed (blown up). |  | - Charge and store the battery pack at a temperature of 0°C to 40°C – avoid a temperature higher than 65°C when discharging. - Do not shortcut – Risk of explosion and fire! |
| --- | --- | --- |

| CE Declaration Herewith we declare that this item is in accordance with the essential requirements and other relevant regulations of the directive 2004/108/EC. A copy of the original declaration of conformity can be obtained at the address above. |  | WEEE Reg. Nr.: DE64989527 Information for disposal of crossed Bin marked products This symbol is marked on a product or packaging, it means that the product including the batteries must not be disposed of with your general household waste. Only discard electrical/electronic items in separate collection schemes, which cater for the recovery and recycling of materials contained within. Your co-operation is vital to ensure the success of these schemes and for the protection of the environment. |
| --- | --- | --- |
| This product uses Li-Ion batteries. Li-Ion batteries are recyclable, and a valuable resource. To dispose of the battery, remove it and take it to a resource recovery facility. |  |  |

ATTENTION! The privacy of others is protected by law and should be taboo. The recording of third party can be an illegal act. You are responsible for your acting. Electrostatic discharge or strong electromagnetic fields can affect the Camera. Restart the Camera in this case.

Content of Box
Check the completeness before using the product

CamOne Infinity Camera 1080p

USB-Cable           USB –AV-Cable          Ten-2-Ten Adaptor          Battery

DiveBox	      Long connector	      Short connector           Bike Mount

M-Mount                               Helmet Mount                             Universal Mount

Velcro Pad small 	        Velcro Pad big 	            Manual

Preparation

Compensation for recorded content

ACME will not be responsible in any case for any consequential or direct problems that will lead to a loss of recorded or edited data. Furthermore ACME will not guarantee the content in case the recording or saving is not working properly. The above limitations are as well valid in case the unit has been repaired.

Do not remove the micro SD Card while recording or saving. Do not switch off or take out the battery before the recording has been stopped or as long there is access to the memory, indicated by a rotating circle.

Protect the unit from extreme temperatures. Stop recording at temperatures less than minus 10°C and higher than 60°C. Remove the camera from the windshield/sucking pad in case extreme temperatures might be possible!

It is known that forged accumulators which remarkably look like the original product are offered for the sale again and again. Some of these accumulators are not equipped with a proper protection circuit which would satisfy the regulations of the prevailing safety regulations. These accumulators can perhaps lead to fires or explosions. We assume no liability for accidents, damages or failures which result from the use of a forged accumulator.

Surface of the camera
The shell of the camera is made of high quality aluminum, an energy-consuming process. Accepting tiny superficial casting defects, not affecting the quality of the camera, reduces the reject rate by 80%.

Insert the battery

Push up the lock battery compartments lock and remove the cover. Below this you will find the battery case. Insert the battery by following the golden contacts inside the cam.

Only use the original battery (Art-No.: COIN08).

Batteries of other suppliers may have different polarities and cause damages!

Charging the battery

The inside battery is not fully charged when being purchased.  Please charge the battery before you are going to use the CamOne Infinity the first time!
Connect the camera with your computer using the included USB-cable. The charging process is indicated by the red LED light which switches to green as soon as charging is finished.
Charging takes around 2 hours.

You can alternately use the 5V USB-Power adaptor (Art. No.: FCHD43).

Functional diagram

The USB comport has the following setup:

-AV-Out

-5V IN
-Signal IN for external Start/Stop (CarCamOne-Cable / Motion sensor)

-GPS-Module / Video transmitter Set data input

Due to limitations in the technology of the display production, single or few dark or light dots might appear. This is not a defect and will not have any impact to the recorded video or photo!

Key Panel – Interface Overview

| Compatible cards |  |
| --- | --- |
|  |  | Only use Micro-SDHC-Cards rank „Class4“ or higher. The camera can be used with cards up to 32GB. The recommended cards are "FCO Rapid Rush" cards at a minimum size of 8GB. Using a 32GB card allows a recording time of 4,5 hours at highest resolution. |

Insert/ Remove Memory Card

|  | Remove the comport cover protecting the sockets and the card slot. Slide in the card upside down - printed side to the bottom - until it clicks in. Use Slot 1 at first. If you are using 2 cards, the cam automatically switches to the second card as soon as the memory is full. To remove press onto the visible edge of the card, so it will get out by the spring mechanism. |
| --- | --- |

Turn on the camera

|  | On/Off Press the power button on the top of the cam until it turns on. |
| --- | --- |

Icons at the Display

Due to the different configurations, you will see various icons on the display:

SETUP Menu

Please press the M- Button for 3 seconds to enter the setup.

The menu is divided in 3 categories that can be selected by the different tabs (Photo, Video and system settings). By pressing the M- Button you will enter the next tab.

|  | Photo-Menu Here you can select all necessary settings for photo recordings. |
| --- | --- |
| -Picture quality: -Photo Intervals: -Serial Picture: -Time stamp: | Choice between 3, 5 or 8 Mega Pixel Off, quick recording of 3, 5 or 10 pictures directly behind each other Off, pictures will be shot every 3 Sec., 5 Sec., 10Sec., 30 Sec. or 60 Sec. On or Off (embedded in the picture) |
|  | Video-Menu Here you can select all necessary settings for video recordings. |
| -Video Quality: -Recording Quality: -Recording: -Time stamp: -Time lapse: -Loop Mode: -Microphone: | Resolution ¦ Videoformat PAL / NTSC: 480p (720 x 480 Pixel) ¦ 25 / 30 FPS 720p (1280 x 720 Pixel) ¦ 25 / 30 FPS 720pH (1280 x 720 Pixel) ¦ 50 / 60 FPS 1080ps (1440 x 1080 Pixel) ¦ 25 / 30 FPS 1080p (1920 x 1080 Pixel) ¦ 25 / 30 FPS Low, Medium or High Duration of the recording. 5, 10, 15, 20 or 30 Minutes. Due to the data volumes a longer recording is not possible. On or Off (embedded in the video) Off, 1 frame per Second (FPS), 6 frames per minute (FPM), 2 FPM or 1 FPM Overwriting of older recordings on or off (in case of storage space is wasted) On or off |
|  | System Settings Here you can choose all general settings. |
| -Languages: -Date: -Time: -Auto Screen off: -Auto Power off: -Motion detection: -Flip picture: -Image mirroring: -Light Frequency: -Video System: -Status LED: -Card formatting: -Factory setting -Firmware: | Choose between German, English, French, Spanish, Italian or Dutch Setting the date (Year-Month-Day) Setting the time (Hour : Minute) Battery saving to be selected here: Off, 30 Sec., 60 Sec. or 120 Sec. Select the Auto off function: Off, 60 Sec., 120 Sec. or 300 Sec. Off, low, medium or high normal or 180° flipped normal or mirrored 50/60Hz - Before you start recording indoor, choose the correct frequency: Europe, Asia and Australia: 50Hz America: 60Hz Japan: Depending on the area 50 or 60Hz NTSC or PAL On or Off of the front LEDs Choice between Slot 1 or Slot 2 (All recorded data will be deleted!) Resetting the camera Information about current firmware and update |

Please note that some functions are only available after a firmware-/ software update.

Using the DiveBox and different mounts

Please make sure that the lens of the cam and also the lens-protection of the inner side of the box is clean.

Insert the cam. Please check carefully that the fastener of the box is properly locked. The cam can still be operated though the buttons.

When using the bike or helmet mount, please pay attention that the adjusting screws are fixed tightly.

Mounts that are glued on must have a clean surface.

Photo/ Video recording

|  |  |  | Right after switching on the Cam it is possible to take pictures or videos by pressing the ’record’ button. Press the button again to stop the recording. Never remove the SD-card or the battery during the data-saving. (shown with a rotating circle in the display). |  |
| --- | --- | --- | --- | --- |
| Time lapse recording |  |  |

Press the „M-Button“ to enter the setup menu. Choose the video menu and then „Time lapse“.

You can choose between: 1 FPS, 6 FPM, 2 FPM or 1 FPM.

Confirm the requested function and leave the menu.

By pressing the record button you can start the video. By pressing this button again the recording is stopped and will be saved on the memory card. If you are planning to record for a longer time, we recommend to turn off the screen in order to save power (Menu: „Auto Screen off“).

Photo Serial Recording

Press the „M-Button“ to enter the Setup Menu. Choose the photo menu and then „Serial Picture“.

You can choose between: 3 Sec., 5 Sec., 10Sec., 30 Sec. or 60 Sec. Confirm the requested function and leave the menu.

By pressing the record button photos will be shot. By pressing this button again the recording is stopped and will be saved as JPG on the memory card.

Photo Interval

Press the „M-Button“ to enter the Setup Menu. Choose the photo menu and then „Photo Interval“.

You can choose between: 3, 5 or 10 photos.

Confirm the requested function and leave the menu.

By pressing the record button the camera will release the intervals. The photos will be saved as JPG on the card.

Using the zoom (only NTSC)

|  |  | During the recording (Video) in mode 720p you can Zoom In and Out. The zoom will be shown by a status bar in the display. Zooming in 1080p and PAL mode is not possible. By pressing the Video or Photo button you can choose the zoom factor. |
| --- | --- | --- |

Playback of Videos and Photos

To watch your recorded data, press the „Play“ button.

You have the choice between photos and videos. Choose this with the Photo and Video button.

Confirm your choice by pressing the Play button.

To playback the recordings, press „Play“. The display shows the name of the files.

Playback on the TV

Connect the camera with your TV by using a HDMI Cable and switch to the corresponding channel. Enter the Playback Mode on the camera to watch your videos.

Playback on PC (MS Windows)

The videos can be easily watched on your PC.

We recommend the free VLC- Player.

This can be downloaded on www.vlc-player.org

Connect the camera to your PC by using the USB- Cable. Turn on the camera.

Choose the card access with the buttons „Photo and Video“ and confirm with the Play button. The PC will recognize the cam as a hard drive.

The display will show „Connect to PC“ for a short time.

Usage of external Video Transmission / Screen / Motion Sensor / GPS (optional)

With the supplied Ten2Ten Adaptor you can easily connect the camera with many accessories of the FlyCamOne HD family. Plug in the adaptor to the USB plug of the cam and connect the accessories. The following products are available (optional): -GPS Module (Art. No. FCHD15) Indication of various GPS-Data -2,5“ external screen (Art. No. FCHD02) Watch the video on the bigger display. The camera display shuts off automatically. -Video Transmission 5,8GHz (Art. No. FCHD03) Transmit the picture on the screen or the video goggles. The camera display shuts off automatically. For more detailed advice please contact the homepage www.CamOneTec.com

| Replacing the lens (optional) |  |
| --- | --- |
| FPV-Kopf für den |  | To replace the lens, twist off the two screws on the picture shown. Unscrew the lens. Please pay attention that no dust or dirt get into the cam. Screw in the replacement lens. Check the sharpness with a TV. Fix the cam with the two screws. |
| Please note! Inappropriate opening may damage the camera head and ports. Perform this work very carefully and take your time! LinX 2.4GHz Digital Video Transmission & Remote (optional). In conjunction with the optional bi-directional LinX 2.4GHz digital transmitter (Art.-No.:COIN10) you can control the camera up to a distance of 300 meters (on view) and watch the video image also. Obstacles may affect the range. The LinX System is able to control several cameras and supports also the direction of the cam using the looka-O-module. |  |

Firmware Update

To get the most out of your camera, install the latest software (Firmware).

The firmware version of your camera is shown in the system settings.

Download the ZIP file from our homepage first.

Unzip the file on the main root of the memory card. Enter the system settings of the camera and choose Firmware Update -> "Yes" and confirm with the “Play”- button.

This process takes about 1 minute. The display shows the procedure.

Do not turn off the cam while updating!
Make sure that the battery is fully charged.

Technical Data

Operating Voltage:	5V (USB) / 3,7V (LiPo-Battery, 900mAh)

Consumption:	250 - 450mAh (depending on camera settings)

Size: 	49 x 42 x 34mm

Weight:	88g incl. battery

Battery:	3,7V 900mAh LiPo
Format:	MPEG4, AVI or MOV

Sensor:	5MPix CMOS

Lens:	170° (exchangeable)

Zoom:	digital 4times (only NTSC)

Screen:	1,5“ / 38mm TFT

Microphone:	yes (Mono, 48 kHz)

AV-Output:	yes, via USB-10 Pin

USB:	Hi-Speed USB (2.0)

HDMI:	yes, 1.2

Interface:	Open Connector Board (OCB, bottom of the cam)
Remote:	yes, LinX-System 2,4 GHz (optional)

Operating-/Storagetemp.:	 -10°C till 60°C

Memory:	Micro SD/SDHC-<= 32GB – Class 4 or higher
	2 slots for cards with 32GB each

White balance:	automatically

Troubleshooting

| Error | Reason | Solution |
| --- | --- | --- |
| Camera switches off immediately. | Battery is not charged correctly. | Charge the camera for at least 4 hours. |
| Unable to switch ON the camera | Battery is not plugged correctly. | Check connection between camera and battery. |
|  | Battery is defective. | Replace the battery. |
|  | The camera had a hang- up during recording. | Remove the battery for 10 seconds. |
| The Video jerks when playing at the PC. | The USB- connection is too slow. | Download the file to the PC first. |
|  | The micro SD- Card is too slow. | Try one of the recommended micro SD- Cards. |
|  | The computer is too slow | Connect the camera and your TV with the AV- Cable. |
|  | The data rate is too high for the computer. | Reduce data rate to “Standard”." |
| The video is turned 180° | Wrong Settings | Use setup configuration To turn the picture |
| Date and time will not be displayed in a correct way | Wrong Settings | Change Settings in the Setup |
| Display shows „Error“ | The memory card is not working correctly | Format the memory card. Card is possibly damaged. |

CamOne Infinity HD Accessories

| Art-No.: | EAN | Item |
| --- | --- | --- |
| FCHD02 | 4260033 049520 | FlyCamOne HD Screen 2,5"; 150cm cable |
| FCHD03 | 4260033 049537 | FlyCamOne HD Transmission Set 5.8GHz; Range 300m |
| COIN08 | 4260033041975 | CamOne Infinity Battery LiPo 3,7V 900mAh |
| COIN10 |  | CamOne LinX 2.4GHz Digital Remote |
| FCHD15 | 4260033049650 | FlyCamOne HD GPS Module |
| FCHD16 | 4260033 049667 | FCHD Rapid Rush 16GB Micro SD-Card |
| FCHD32 | 4260033 049827 | FCHD Rapid Rush 32GB Micro SD-Card |
| FCHD43 | 4260033048578 | USB Power adaptor 110-220V |
| FC3104 | 4260033 048721 | 5.8GHz Shield antenna + 500m |

...get inside at www.camonetec.com